RFID :

### Components:

- Arduino UNO

- RFID Module (e.g., RC522)

- Servo Motor

- Jumper wires

- Breadboard (optional)

- Lock (servo motor simulating a lock)

### Wiring the components:

- RFID Module (RC522) to Arduino UNO:

  - SDA (RFID) -> Pin 10 (Arduino)

  - SCK (RFID) -> Pin 13 (Arduino)

  - MOSI (RFID) -> Pin 11 (Arduino)

  - MISO (RFID) -> Pin 12 (Arduino)

  - IRQ (RFID) -> Not connected

  - GND (RFID) -> GND (Arduino)

  - RST (RFID) -> Pin 9 (Arduino)

  - 3.3V (RFID) -> 3.3V (Arduino)

- Servo Motor to Arduino UNO:

  - Servo signal (PWM pin) -> Pin 6 (Arduino)

  - VCC (Servo) -> 5V (Arduino)

  - GND (Servo) -> GND (Arduino)

### Arduino Code:

cpp

CopyEdit

#include <SPI.h>

#include <MFRC522.h>

#include <Servo.h>

// Define RFID pins

#define SS_PIN 10

#define RST_PIN 9

MFRC522 mfrc522(SS_PIN, RST_PIN);

// Define Servo motor pin

Servo lockServo;

int servoPin = 6;

int lockedPosition = 0;  // Locked position of the servo

int unlockedPosition = 90;  // Unlocked position of the servo

void setup() {

// Start serial communication for debugging

Serial.begin(9600);

// Initialize RFID module

SPI.begin();

mfrc522.PCD_Init();

// Initialize the servo motor

lockServo.attach(servoPin);

lockServo.write(lockedPosition);  // Start with the door locked

Serial.println("Scan your RFID card/tag to unlock the door.");

}

void loop() {

// Look for new RFID cards

if (mfrc522.PICC_IsNewCardPresent()) {

if (mfrc522.PICC_ReadCardSerial()) {

// Print the UID of the card/tag

Serial.print("Card UID: ");

String content = "";

for (byte i = 0; i < mfrc522.uid.size; i++) {

content.concat(String(mfrc522.uid.uidByte[i] < 0x10 ? " 0" : "");

content.concat(String(mfrc522.uid.uidByte[i], HEX));

}

content.toUpperCase();

Serial.println(content);

// Check for a specific UID (for example, UID "04 1F D0 16")

if (content == "04 1F D0 16") {

Serial.println("Access granted, unlocking door...");

lockServo.write(unlockedPosition);  // Unlock the door (move servo)

delay(5000);  // Keep the door unlocked for 5 seconds

lockServo.write(lockedPosition);    // Lock the door after 5 seconds

} else {

Serial.println("Access denied!");

}

}

}

}

### Explanation:

- RFID Module: This reads the UID of the RFID tag. The tag's UID is compared to a pre-programmed value ("04 1F D0 16" in this example), and if it matches, the door unlocks.

- Servo Motor: The servo motor is used to simulate the door lock. It moves to the unlocked position when the correct tag is scanned and then returns to the locked position after a brief delay.

- Arduino: The Arduino controls the communication between the RFID module and the servo motor based on the tag scanned.

### Steps:

- Install Libraries: Ensure you have the MFRC522 and Servo libraries installed in your Arduino IDE.

  - To install the libraries, go to Sketch > Include Library > Manage Libraries... and search for MFRC522 and Servo.

- Upload Code: Upload the code to your Arduino UNO and open the serial monitor to see the RFID card's UID and status messages.

- Testing: Scan a valid RFID tag to unlock the door and see the servo motor's position change.

Let me know if you need more assistance!

4o