Soil Moisture and LDR

// Soil Moisture Sensor Section

// Define the analog input pin for the soil moisture sensor

const int soilMoisturePin = A0;  // Pin connected to the soil moisture sensor

// Define the digital output pin for the LED

const int soilMoistureLedPin = 8;  // Pin connected to the LED

// Define the threshold value for dry soil (adjust this based on your sensor's calibration)

const int drySoilThreshold = 800;

// LDR Sensor Section

// Define the LDR sensor pin and LED pin

const int ldrPin = A1;   // LDR connected to analog pin A1

const int ldrLedPin = 13;  // LED connected to digital pin 13

// Set the threshold for detecting darkness

const int darkThreshold = 500;  // Adjust this value based on testing

// Timing Constants

const int soilMoistureDelay = 1000;  // Delay for soil moisture readings (milliseconds)

const int ldrDelay = 500;           // Delay for LDR readings (milliseconds)

void setup() {

// Initialize serial communication at 9600 baud rate

Serial.begin(9600);

// Set LED pins as outputs

pinMode(soilMoistureLedPin, OUTPUT);

pinMode(ldrLedPin, OUTPUT);

}

void loop() {

// Soil Moisture Sensor Logic

int soilMoistureValue = analogRead(soilMoisturePin);  // Read soil moisture value

Serial.print("Soil Moisture Value: ");

Serial.println(soilMoistureValue);

if (soilMoistureValue > drySoilThreshold) {

digitalWrite(soilMoistureLedPin, HIGH);  // Turn on LED for dry soil

Serial.println("Soil is dry - LED ON");

} else {

digitalWrite(soilMoistureLedPin, LOW);   // Turn off LED for wet soil

Serial.println("Soil is wet - LED OFF");

}

delay(soilMoistureDelay);  // Wait before next soil moisture reading

// LDR Sensor Logic

int ldrValue = analogRead(ldrPin);  // Read LDR value

Serial.print("LDR Value: ");

Serial.println(ldrValue);

if (ldrValue < darkThreshold) {

digitalWrite(ldrLedPin, HIGH);  // Turn on LED to indicate night

Serial.println("Street light on - Night");

} else {

digitalWrite(ldrLedPin, LOW);   // Turn off LED to indicate day

Serial.println("Daytime - Street light off");

}

delay(ldrDelay);  // Wait before next LDR reading

}